# Registro Cisco Packet Tracer

## Comandos Basicos

Criptografar as senhas existentes

- enable

- configure terminal

- service password-encryption

Nome

- enable

- configure terminal

- hostname Roteador

Senha Enable - MODO EXEC

- enable

- configure terminal

- enable password cisco (descriptgrafado) ou enable secret cisco (criptografado)

Senha Console -  MODO USUÁRIO

- enable

- configure terminal

- line console 0

- password cisco

- login

Senha VTY -  Senha Telnet

- enable

- configure terminal

- line vty 0 4

- password cisco

- login (habilita o equipamento a pedir a senha)

Banner

- enable

- configure terminal

- banner motd @Banner@

Interface

- enable

- configure terminal

- interface gigabitEthernet x/y

- ip address xxx.xxx.xxx.xxx xxx.xxx.xxx.xxx

- no shutdown

Salvar Configuração

- enable

- copy running-config startup-config

Ver versão do SO

- enable

- show version

Liberar SSH

- enable

- configura terminal

- Configurar Nome Do Host

- hostname NomeDoHost

- Setar Nome Do Dominio

- ip domain-name cisco.com

- Criar chave criptografa

- crypto key generate rsa (gerar a chave criptografada)

- tamanho da chave : valor padrão usado é 1024

- Criar Usuário

- usarnme nome secret senha

- 8º setar ssh como padrão

- line vty 0 4

- transport input ssh

- login local

- Setar A Versão Mais Recente

- ip ssh version 2

## Topologia IPV4

Roteador 1

- hostname R01

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.208.1 255.255.240.0

- interface GigabitEthernet0/1

- ip address 177.100.128.1 255.255.240.0

- interface GigabitEthernet0/2

- ip address 177.100.144.1 255.255.240.0

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

Roteador 2

- hostname R02

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.224.1 255.255.240.0

- interface GigabitEthernet0/1

- ip address 177.100.223.254 255.255.240.0

- interface GigabitEthernet0/2

- ip address 177.100.160.1 255.255.240.0

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

Roteador 3

- hostname R03

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.176.1 255.255.240.0

- interface GigabitEthernet0/1

- ip address 177.100.239.254 255.255.240.0

- interface GigabitEthernet0/2

- ip address 177.100.192.1 255.255.240.0

- service

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

## Topologia IPV6

Roteador 1

- hostname R01

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.208.1 255.255.240.0

- ipv6 address 2005:db8:d400::/38

- no shutdown

- interface GigabitEthernet0/1

- ip address 177.100.128.1 255.255.240.0

- ipv6 address 2005:db8:c000::/38

- no shutdown

- interface GigabitEthernet0/2

- ip address 177.100.144.1 255.255.240.0

- ipv6 address 2005:db8:c400::/38

- no shutdown

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

- ip route 177.100.160.0 255.255.240.0 177.100.223.254

- ip route 177.100.176.0 255.255.240.0 177.100.223.254

- ip route 177.100.192.0 255.255.240.0 177.100.223.254

- ip route 177.100.224.0 255.255.240.0 177.100.223.254

Roteador 2

- hostname R02

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.224.1 255.255.240.0

- ipv6 address 2005:db8:d800::/38

- no shutdown

- interface GigabitEthernet0/1

- ip address 177.100.223.254 255.255.240.0

- ipv6 address 2005:db8:d7ff:ffff:ffff:ffff:ffff:ffff/38

- no shutdown

- interface GigabitEthernet0/2

- ip address 177.100.160.1 255.255.240.0

- ipv6 address 2005:db8:c800::/38

- no shutdown

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

- ip route 177.100.128.0 255.255.240.0 177.100.208.1

- ip route 177.100.144.0 255.255.240.0 177.100.208.1

- ip route 177.100.176.0 255.255.240.0 177.100.239.254

- ip route 177.100.192.0 255.255.240.0 177.100.239.254

Roteador 3

- hostname R03

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.176.1 255.255.240.0

- ipv6 address 2005:db8:cc00::/38

- no shutdown

- interface GigabitEthernet0/1

- ip address 177.100.239.254 255.255.240.0

- ipv6 address 2005:db8:dbff:ffff:ffff:ffff:ffff:ffff/38

- no shutdown

- interface GigabitEthernet0/2

- ip address 177.100.192.1 255.255.240.0

- ipv6 address 2005:db8:d000::/38

- no shutdown

- service

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

- ip route 177.100.128.0 255.255.240.0 177.100.224.1

- ip route 177.100.144.0 255.255.240.0 177.100.224.1

- ip route 177.100.208.0 255.255.240.0 177.100.224.1

- ip route 177.100.160.0 255.255.240.0 177.100.224.1

## Topologia IPv4 e IPv6 - Roteamento Estático

Roteador 1

- hostname R01

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.208.1 255.255.240.0

- ipv6 address 2005:db8:d400::/38

- no shutdown

- interface GigabitEthernet0/1

- ip address 177.100.128.1 255.255.240.0

- ipv6 address 2005:db8:c000::/38

- no shutdown

- interface GigabitEthernet0/2

- ip address 177.100.144.1 255.255.240.0

- ipv6 address 2005:db8:c400::/38

- no shutdown

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

- ip route 177.100.160.0 255.255.240.0 177.100.223.254

- ip route 177.100.176.0 255.255.240.0 177.100.223.254

- ip route 177.100.192.0 255.255.240.0 177.100.223.254

- ip route 177.100.224.0 255.255.240.0 177.100.223.254

- ipv6 route 2005:DB8:C800::/38 2005:DB8:D7FF:FFFF:FFFF:FFFF:FFFF:FFFF

- ipv6 route 2005:DB8:D800::/38 2005:DB8:D7FF:FFFF:FFFF:FFFF:FFFF:FFFF

- ipv6 route 2005:DB8:CC00::/38 2005:DB8:D7FF:FFFF:FFFF:FFFF:FFFF:FFFF

- ipv6 route 2005:DB8:D000::/38 2005:DB8:D7FF:FFFF:FFFF:FFFF:FFFF:FFFF

Roteador 2

- hostname R02

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.224.1 255.255.240.0

- ipv6 address 2005:db8:d800::/38

- no shutdown

- interface GigabitEthernet0/1

- ip address 177.100.223.254 255.255.240.0

- ipv6 address 2005:db8:d7ff:ffff:ffff:ffff:ffff:ffff/38

- no shutdown

- interface GigabitEthernet0/2

- ip address 177.100.160.1 255.255.240.0

- ipv6 address 2005:db8:c800::/38

- no shutdown

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

- ip route 177.100.128.0 255.255.240.0 177.100.208.1

- ip route 177.100.144.0 255.255.240.0 177.100.208.1

- ip route 177.100.176.0 255.255.240.0 177.100.239.254

- ip route 177.100.192.0 255.255.240.0 177.100.239.254

- ipv6 route 2005:DB8:C000::/38 2005:DB8:D400::

- ipv6 route 2005:DB8:C400::/38 2005:DB8:D400::

- ipv6 route 2005:DB8:CC00::/38 2005:DB8:DBFF:FFFF:FFFF:FFFF:FFFF:FFFF

- ipv6 route 2005:DB8:D000::/38 2005:DB8:DBFF:FFFF:FFFF:FFFF:FFFF:FFFF

Roteador 3

- hostname R03

- enable secret cisco

- interface GigabitEthernet0/0

- ip address 177.100.176.1 255.255.240.0

- ipv6 address 2005:db8:cc00::/38

- no shutdown

- interface GigabitEthernet0/1

- ip address 177.100.239.254 255.255.240.0

- ipv6 address 2005:db8:dbff:ffff:ffff:ffff:ffff:ffff/38

- no shutdown

- interface GigabitEthernet0/2

- ip address 177.100.192.1 255.255.240.0

- ipv6 address 2005:db8:d000::/38

- no shutdown

- service

- banner motd @Acesso Restrito! Caia Fora seu Lixo!@

- line con 0

- password class

- login

- line aux 0

- line vty 0 4

- password class

- login

- service password-encryption

- ip route 177.100.128.0 255.255.240.0 177.100.224.1

- ip route 177.100.144.0 255.255.240.0 177.100.224.1

- ip route 177.100.208.0 255.255.240.0 177.100.224.1

- ip route 177.100.160.0 255.255.240.0 177.100.224.1

- ipv6 route 2005:DB8:C800::/38 2005:DB8:D800::

- ipv6 route 2005:DB8:C400::/38 2005:DB8:D800::

- ipv6 route 2005:DB8:C000::/38 2005:DB8:D800::

- ipv6 route 2005:DB8:D400::/38 2005:DB8:D800::

## Protocolo EIGRP

Roteador R1

- configure terminal

- router eigrp 1

- show ip route

- network 172.16.1.0 0.0.0.255

- network 172.16.3.0 0.0.0.3

- network 192.168.10.4 0.0.0.3

- passive-interface GigabitEthernet0/0

- no auto-summary

Roteador R2

- configure terminal

- router eigrp 1

- show ip route

- network 172.16.1.0 0.0.0.255

- network 172.16.3.0 0.0.0.3

- network 192.168.10.4 0.0.0.3

- passive-interface GigabitEthernet0/0

- no auto-summary

Roteador R3

- configure terminal

- router eigrp 1

- show ip route

- network 172.16.1.0 0.0.0.255

- network 172.16.3.0 0.0.0.3

- network 192.168.10.4 0.0.0.3

- passive-interface GigabitEthernet0/0

- no auto-summary

## Configuring RIPv2

Roteador R1

- configure terminal

- router rip

- version 2

- no auto-summary

- network 172.31.10.0

- network 172.31.20.0

- network 172.31.30.0

- passive-interface GigabitEthernet0/0

- default-information originate

Roteador R2

- configure terminal

- router rip

- version 2

- no auto-summary

- network 172.31.10.0

- network 172.31.20.0

- network 172.31.30.0

- passive-interface GigabitEthernet0/0

- default-information originate

Roteador R3

- configure terminal

- router rip

- version 2

- no auto-summary

- network 172.31.10.0

- network 172.31.20.0

- network 172.31.30.0

- passive-interface GigabitEthernet0/0

- default-information originate

## Configuring OSPFv2 in a Single Area

Roteador R1

- configure terminal

- router ospf 10

- router-id 172.16.1.1

- network 172.16.3.1 0.0.0.0 area 0

- network 172.31.20.0 0.0.0.255 area 0

- network 172.31.30.0 0.0.0.255 area 0

- passive-interface GigabitEthernet0/0

- exit

Roteador R2

- configure terminal

- router ospf 10

- router-id 2.2.2.2

- network 172.31.10.0 0.0.0.255 area 0

- network 172.31.20.0 0.0.0.255 area 0

- network 172.31.30.0 0.0.0.255 area 0

- passive-interface GigabitEthernet0/0

- exit

Roteador R3

- configure terminal

- router ospf 10

- router-id 3.3.3.3

- network 172.31.10.0 0.0.0.255 area 0

- network 172.31.20.0 0.0.0.255 area 0

- network 172.31.30.0 0.0.0.255 area 0

- passive-interface GigabitEthernet0/0

- exit

## Configuração Vlan

Switch S1

- configure terminal

- vlan 10

- name VLAN_10

- vlan 20

- name VLAN_20

- vlan 30

- name VLAN_30

- vlan 99

- name VLAN_99

- vlan 150

- name VLAN_150

Verificar a Configuração de VLAN

- show vlan brief

Switch S2

- configure terminal

- vlan 10

- name VLAN_10

- vlan 20

- name VLAN_20

- vlan 30

- name VLAN_30

- vlan 99

- name VLAN_99

- vlan 150

- name VLAN_150

Atribuir VLANs às Portas

- configure terminal

- interface FastEthernet0/11

- switchport mode access

- switchport access vlan 10

- interface FastEthernet0/18

- switchport mode access

- switchport access vlan 20

- interface FastEthernet0/6

- switchport mode access

- switchport access vlan 30

Verificar a Configuração de VLAN

- show vlan brief

Switch S3

- configure terminal

- vlan 10

- name VLAN_10

- vlan 20

- name VLAN_20

- vlan 30

- name VLAN_30

- vlan 99

- name VLAN_99

- vlan 150

- name VLAN_150

Atribuir VLANs às Portas

- configure terminal

- interface FastEthernet0/11

- switchport mode access

- switchport access vlan 10

- interface FastEthernet0/18

- switchport mode access

- switchport access vlan 20

- interface FastEthernet0/6

- switchport mode access

- switchport access vlan 30

## Configuração Trunk

Switch S1

- configure terminal

- interface GigabitEthernet0/1

- switchport mode trunk

- switchport trunk native vlan 10

- interface GigabitEthernet0/2

- switchport mode trunk

- switchport trunk native vlan 10

Switch S2

- configure terminal

- interface GigabitEthernet0/1

- switchport mode trunk

- switchport trunk native vlan 10

Switch S3

- configure terminal

- interface GigabitEthernet0/2

- switchport mode trunk

- switchport trunk native vlan 10

## Configuração Vlan e Trunk

Switch S1

- configure terminal

- hostname S1

- enable secret cisco

- line console 0

- password class

- login

- line vty 0 4

- password  class

- login

- service password-encryption

- banner motd #Mensagem de acesso não autorizado proibido#

Configuração das VLANs e IP da VLAN 99

- configure terminal

- vlan 10

- name VLAN_10

- vlan 20

- name VLAN_20

- vlan 99

- name Management

- interface vlan 99

- ip address 192.168.1.11 255.255.255.0

- no shutdown

Configuração das Interfaces de Acesso

- configure terminal

- interface FastEthernet0/11

- switchport mode access

- switchport access vlan 10

- interface FastEthernet0/12

- switchport mode access

- switchport access vlan 20

- interface FastEthernet0/18

- switchport mode access

- switchport access vlan 99

Configuração da Porta Trunk (FastEthernet0/1)

- configure terminal

- interface FastEthernet0/1

- switchport mode trunk

- switchport trunk native vlan 99

Switch S2

- configure terminal

- hostname S2

- enable secret cisco

- line console 0

- password class

- login

- line vty 0 4

- password class

- login

- service password-encryption

- banner motd #Mensagem de acesso não autorizado proibido#

Configuração das VLANs e IP da VLAN 99

- configure terminal

- vlan 10

- name VLAN_10

- vlan 20

- name VLAN_20

- vlan 99

- name Management

- interface vlan 99

- ip address 192.168.1.12 255.255.255.0

- no shutdown

Configuração das Interfaces de Acesso

- configure terminal

- interface FastEthernet0/18

- switchport mode access

- switchport access vlan 20

Configuração da Porta Trunk (FastEthernet0/1)

- configure terminal

- interface FastEthernet0/1

- switchport mode trunk

- switchport trunk native vlan 99

## Configure DHCPv4

Roteador R1

- configure terminal

- interface GigabitEthernet0/0

- ip helper-address 10.1.1.2

Roteador R2

- configure terminal

- ip dhcp excluded-address 192.168.10.1 192.168.10.10

- ip dhcp excluded-address 192.168.30.1 192.168.30.10

- dns-server 192.168.20.254

LAN R1

- configure terminal

- ip dhcp pool R1-LAN

- network 192.168.10.0 255.255.255.0

- default-router 192.168.10.1

- dns-server 192.168.20.254

- interface GigabitEthernet0/1

- ip address dhcp

- no shutdown

- show ip dhcp binding

LAN R3

- configure terminal

- ip dhcp pool R3-LAN

- network 192.168.30.0 255.255.255.0

- default-router 192.168.30.1

- Roteador R3

- configure terminal

- interface GigabitEthernet0/0

- ip helper-address 10.2.2.1

## Implement DHCPv4

DHCP

- ip dhcp-client enable

Roteador R1

- configure terminal

- interface GigabitEthernet0/0

- ip address 192.168.10.1 255.255.255.0

- ip helper-address 10.1.1.2

- no shutdown

- exit

Roteador R2

- configure terminal

- ip dhcp excluded-address 192.168.10.1 192.168.10.10

- ip dhcp excluded-address 192.168.30.1 192.168.30.10

- exit

LAN de R1

- configure terminal

- ip dhcp pool R1-LAN

- network 192.168.10.0 255.255.255.0

- default-router 192.168.10.1

- dns-server  192.168.10.254

- exit

LAN de R3

- configure terminal

- ip dhcp pool R3-LAN

- network 192.168.30.0 255.255.255.0

- default-router 192.168.30.1

- dns-server 192.168.30.254

- exit

Roteador R3

- configure terminal

- interface GigabitEthernet0/0

- ip address 192.168.30.1 255.255.255.0

- ip helper-address 10.2.2.2

- no shutdown

- exit

## Skills Integration Challenge

DHCP:

- ip dhcp-client enable

R1

- configure terminal

- interface GigabitEthernet0/0

- no shutdown

- exit

- interface GigabitEthernet0/0.10

- encapsulation dot1Q 10

- ip address 172.31.10.1 255.255.255.224

- ip helper-address 192.168.30.1

- exit

- interface GigabitEthernet0/0.20

- encapsulation dot1Q 20

- ip address 172.31.20.1 255.255.255.240

- ip helper-address 192.168.30.1

- exit

- interface GigabitEthernet0/0.30

- encapsulation dot1Q 30

- ip address 172.31.30.1 255.255.255.128

- ip helper-address 192.168.30.1

- exit

- interface GigabitEthernet0/0.40

- encapsulation dot1Q 40

- ip address 172.31.40.1 255.255.255.192

- ip helper-address 192.168.30.1

- exit

R2

- configure terminal

- ip dhcp excluded-address 172.31.10.1 172.31.10.10

- ip dhcp excluded-address 172.31.20.1 172.31.20.10

- ip dhcp excluded-address 172.31.30.1 172.31.30.10

- ip dhcp excluded-address 172.31.40.1 172.31.40.10

- exit

- ip dhcp pool VLAN_10

- network 172.31.10.0 255.255.255.224

- default-router 172.31.10.1

- dns-server 209.165.201.14

- exit

- ip dhcp pool VLAN_20

- network 172.31.20.0 255.255.255.240

- default-router 172.31.20.1

- dns-server 209.165.201.14

- exit

- ip dhcp pool VLAN_30

- network 172.31.30.0 255.255.255.128

- default-router 172.31.30.1

- dns-server 209.165.201.14

- exit

- ip dhcp pool VLAN_40

- network 172.31.40.0 255.255.255.192

- default-router 172.31.40.1

- dns-server 209.165.201.14

- exit

Switch 2

- configure terminal

- vlan 10

- name VLAN_10

- exit

- vlan 20

- name VLAN_20

- exit

- vlan 30

- name VLAN_30

- exit

- vlan 40

- name VLAN_40

- exit

- interface FastEthernet0/1

- switchport mode trunk

- switchport trunk allowed vlan 10,20,30,40

- exit

- interface FastEthernet0/2

- switchport mode trunk

- switchport trunk allowed vlan 10,20,30,40

- exit

- interface FastEthernet0/3

- switchport mode trunk

- switchport trunk allowed vlan 10,20,30,40

- exit

- interface FastEthernet0/4

- switchport mode trunk

- switchport trunk allowed vlan 10,20,30,40

- exit

- interface FastEthernet0/12

- switchport mode access

- switchport access vlan 10

- exit

- interface FastEthernet0/15

- switchport mode access

- switchport access vlan 20

- exit

- interface FastEthernet0/20

- switchport mode access

- switchport access vlan 30

- exit

- interface FastEthernet0/5

- switchport mode access

- switchport access vlan 40

- exit